PyPBEE User Manual

Version: 1.0
Author: Angshuman Deb
License: MIT
Repository: https://github.com/angshuman311/PyPBEE

# 1. Introduction

PyPBEE is an open-source Python framework designed for performance-based earthquake engineering (PBEE) research. It enables probabilistic analyses across all stages of the PBEE methodology—from seismic hazard to loss assessment—through a modular, object-oriented design.

The framework supports custom model integration, uncertainty quantification, high-performance computing (HPC) execution, and direct interfacing with finite element (FE) platforms such as OpenSeesPy and OpenSeesTcl.

This manual provides step-by-step instructions for installing, configuring, and running PyPBEE using the supplied Bridge Column Example, which demonstrates the full PBEE workflow:

PrelimAnalysis → PSHA → GMS → NLTHA → PSDemHA → PSDamHA

The architecture, mathematical formulation, and theoretical background are described in the companion journal paper.
This manual focuses on practical usage and reproducibility.

# 2. Installation and Setup

## 2.1 System Requirements

- Python: Version 3.9 or later

- Supported Platforms: Windows, macOS, Linux

- Dependencies: NumPy, SciPy, Matplotlib, Pathos, Benedict, PyGMM, OpenSeesPy, Pickle

## 2.2 Installation

Option 1: pip

pip install pypbee

Option 2: Conda (recommended for Windows / HPC users)

conda env create -f environment.yml

conda activate pypbee

- 2.3 Verifying the Installation

python -c "import pypbee; print('PyPBEE imported successfully!')"

If you encounter dependency errors, ensure your environment has matching versions of Numpy (≥1.21), Scipy (≥1.9), and Pathos (≥0.3.0).

3. Example Directory Structure

The example files provided in this release demonstrate the complete workflow for a Caltrans Ordinary Standard Bridge (OSB) column system.

PyPBEE/

│

├── analysis_driver.py                # Executes all PBEE stages

├── create_objects.py                 # Defines inputs and creates analysis objects

├── extract_single_nltha.py           # Extracts and plots results from one NLTHA run

├── ops_model.py                      # OpenSees model definition (parametric form)

├── osb_info_Bridge_A.py              # Model and random variable definitions

│

├── /pypbee                          # PyPBEE source library (core modules)

│     ├── analysis.py

│     ├── psha.py, gms.py, nltha.py

│     ├── psdemha.py, psdamha.py

│     ├── structure.py, im.py, edp.py, ds.py

│     ├── structural_analysis_platform.py

│     └── utility.py, mixture.py, multivariate_nataf.py, ...

│

└── /examples                         # Output directory created after running analysis

4. Running the Bridge Column Example

The bridge column example demonstrates the full probabilistic PBEE workflow, starting from hazard computation to damage hazard assessment.

- Step 1: Define Inputs and Create Objects

File: create_objects.py

This script imports the core PyPBEE classes and defines:

- Bridge geometry, location, and model parameters (osb_info_Bridge_A.py)

- Random variable distributions and correlations

- Structural analysis platform (OpenSeesPy or OpenSeesTcl)

- Intensity measure (e.g., SaT or AvgSa)

- EDPs, DSs, and hazard configurations

Example snippet:

from pypbee.sa_t import SaT

from pypbee.prelim_analysis import PrelimAnalysis

from pypbee.psha import PSHA

from pypbee.gms import GMS

from pypbee.nltha import NLTHA

from pypbee.psdemha import PSDemHA

from pypbee.psdamha import PSDamHA

All objects are instantiated here and imported by the main driver script.

- Step 2: Execute the Full Workflow

File: analysis_driver.py

Run this script from your Python terminal:

python analysis_driver.py

This will execute the complete sequence:

- Preliminary Analysis (PrelimAnalysis)

  - Computes modal properties (eigenvalues, mode shapes).

  - Populates model realization directories.

- Probabilistic Seismic Hazard Analysis (PSHA)

  - Uses the defined ground-motion model (e.g., Boore-Atkinson 2008)

  - Computes site-specific hazard curves for selected IMs.

- Ground Motion Selection (GMS)

  - Selects and scales ground motions consistent with target spectra.

  - Supports stochastic ensemble selection with or without conditioning.

- Nonlinear Time-History Analysis (NLTHA)

  - Runs OpenSees simulations using the parametric ops_model.py.

  - Executes in parallel (local multiprocessing or HPC).

  - Stores EDP time histories and summary results.

- Probabilistic Seismic Demand Hazard Analysis (PSDemHA)

  - Integrates NLTHA demand outputs with IM hazard to derive demand hazard curves.

- Probabilistic Seismic Damage Hazard Analysis (PSDamHA)

  - Convolves demand distributions with fragility functions.

  - Produces damage state hazard and exceedance rates.

- Step 3: Output Files and Results

After successful execution, the following directory structure will appear:

/Results

│

├── Prelim_Analysis_Results/

│   ├── mode_shape_*.txt

│   └── periods.txt

│

├── PSHA_Results/

│   └── psha_results.pickle

│

├── GMS_Results/

│   └── selected_ground_motions/

│

├── NLTHA_Results/

│   ├── node_disp.txt

│   ├── element_forces.txt

│   └── edp_summary.pickle

│

├── PSDemHA_Results/

│   └── demand_hazard_curves/

│

└── PSDamHA_Results/

└── damage_hazard_curves/

You can visualize results using the built-in plotting utilities:

edp.plot_conditional_demand_model_3d(...)

ds.plot_mrp_histogram(...)

im.plot_shc(...)

5. Customizing and Extending PyPBEE

PyPBEE is built for extensibility. Advanced users can easily add new entity classes or modify workflow behavior.

- 5.1 Adding a Custom IM

Define a new subclass in im.py:

class SaCustom(Sa):

def evaluate_period(self, for_which):

return custom_period_function(for_which)

- 5.2 Adding a Custom EDP

Create a new class in edp.py:

class MaxFiberStrain(EDP):

def generate_recorder(self):

# define fiber strain recorders for section/fiber IDs

pass

def evaluate_edps(self):

# compute strain range or envelope

pass

- 5.3 Plug-and-Play UQ Integration

Users can replace PyPBEE’s internal sampling algorithms by overriding:

Structure.generate_model_param_vals()

For example, implementing Variance-Targeted Population Monte Carlo (VT-PMC) or MCMC samplers.

6. Uncertainty Quantification (UQ)

PyPBEE handles both aleatory and epistemic uncertainties:

- Aleatory: Variability in seismic hazard, ground motion, and FE model parameters.

- Epistemic: Limited data or model-form uncertainty.

The current UQ layer supports:

- MultivariateNataf for correlated variables

- Mixture distributions for compound uncertainty

- Plug-in architecture for user-defined sampling functions

Example:

from pypbee.multivariate_nataf import MultivariateNataf

nataf = MultivariateNataf(distributions, correlation_matrix)

samples = nataf.sample(n=1000)

7. Troubleshooting

| Issue | Possible Cause | Suggested Fix |
| --- | --- | --- |
| ImportError | Environment not activated | Activate with conda activate pypbee |
| OpenSeesPy fails to run | Wrong interpreter path | Update path in create_objects.py |
| NLTHA stops early | Numerical divergence | Check ops_model.py constraints or damping |
| No outputs generated | Missing directory permissions | Run with admin rights or correct file paths |
| Multiprocessing hang | Running in IPython | Execute analysis_driver.py from terminal |

8. References

- Moehle & Deierlein (2004). A Framework Methodology for PBEE.

- Porter (2003). PEER PBEE Methodology Overview.

- Jalayer & Cornell (2009). EESD – Probabilistic Seismic Assessment.

- Deb et al. (2021). Advancements in Strain-Based PBEE Methodology.

9. Contact and Support

Lead Developer:
Angshuman Deb
Email: [angshumandeb311@gmail.com]
Affiliation: Degenkolb Engineers